| 53В30P GEAR HOBBING SEMIAUTOMATIC MACHINES вввмммммвииии | VISTAN | # JOINT STOCK # COMPANY «VISTAN» |
| --- | --- | --- |
|  | The semiautomatic machines with upright axis of workpiece for treatment of spur and helical gears, sprockets, worm wheels with hobs by method of form-generating. CE |  |
|  | Delivery terms: FCA Vitebsk (Incoterms 2010) Payment terms: L/C, 100% prepayment HS code 8461401900 Price: 53B30P – 87600 USD, 53B30P-22 – 104000 USD Net weight – 5100 kg # 210627, Republic of Belarus, Vitebsk, Dimitrova str, 36/7 # Tel. + 375-212-370926, 370972, 376673 ### Fax +375-212-363153, 364284 www.vistan.ru e-mail: vistan_ves@yahoo.com |  |

53В30P				GEAR   HOBBING  SEMIAUTOMATIC MACHINE

53В30P-22					STRUCTURAL FEATURES:

- The  semiautomatic  machine  53В30P has a configuration with upright axis of workpiece and with movable table. The  hydraulic  cylinder of blanks clamping is built into the tables spindle.

- The semiautomatic machine 53В30P is equipped with the relay control system.

- The  semiautomatic machine 53В30P-22 is equipped with the SIEMENS programmable controller and drives.

- Adjustment of rotational speed of hob spindle as well as of values of axial and radial feeds is realized with frequency converters-powered asynchronous or  servomotors (depending on version) steplessly.

- High static and dynamic rigidity due to rational shape of column and support carriage guides as well as rigidity increase of main shape-creating units.

- To use full length of  hob cutting edge its periodical travel along the axis (shifting) is provided.

- It is possible to install the tangential support on the semiautomatic machines to realize tangential feed along the axis of hob.

- The  semiautomatic machine  have a centralized lubrication system.

TECHNICAL DATA

|  | 53В30P (-22) |
| --- | --- |
| Maximal outer diameter of gear to be cut, mm | 320 |
| Maximal module to be treated, mm | 6 |
| Maximal helix angle of teeth to be cut, deg. | 60 |
| Maximal width of spur gear face to be cut, mm | 220 |
| Diameter of table, mm | 250 |
| Distance between the tool axis and the table axis, mm: minimal/maximal | 30…250 |
| Maximal diameter of hobs to be installed, mm |  |
| - diameter | 160 |
| - length | 160, 220* |
| Maximal hob travel along the axis (shifting), mm | 75 |
| Range of numbers of hob revolutions, rpm | 50…500 |
| Range of upright feeds, mm/rev.; mm/min. | 0,63…7,3 |
| Range of radial feeds, mm/min. | 1…16 |
| Range of tangential feeds, mm/rev. | 0,26…2,3 |
| Power of two-speed primaryicolumm000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000 motion drive, KW | 3,2/4,2; 7,5* (4,0/4,75) |
| Number of coordinates to be controlled |  |
| Overall dimensions, mm | 2300x1300x1950 |
| Weight of the semiautomatic machine, kg | 5100 |
| * - on special order |  |